DIGITAL SYSTEM DESIGN APPLICATION

EHB436E 	CRN: 11280

Salih Ömer Ongün

040220780

Experiment 2

DECODER

Decoder Truth Table

| I3 | I2 | I1 | I0 | O0 | O1 | O2 | O3 | O4 | O5 | O6 | O7 | O8 | O9 | O10 | O11 | O12 | O13 | O14 | O15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

Design Sources

MSI_Library file design source

module DECODER

(

input [3:0] IN,

output [15:0] OUT

);

reg [15:0] out_reg;

always@(*)

begin

case(IN)

4'b0000: out_reg = 16'b0000_0000_0000_0001;

4'b0001: out_reg = 16'b0000_0000_0000_0010;

4'b0010: out_reg = 16'b0000_0000_0000_0100;

4'b0011: out_reg = 16'b0000_0000_0000_1000;

4'b0100: out_reg = 16'b0000_0000_0001_0000;

4'b0101: out_reg = 16'b0000_0000_0010_0000;

4'b0110: out_reg = 16'b0000_0000_0100_0000;

4'b0111: out_reg = 16'b0000_0000_1000_0000;

4'b1000: out_reg = 16'b0000_0001_0000_0000;

4'b1001: out_reg = 16'b0000_0010_0000_0000;

4'b1010: out_reg = 16'b0000_0100_0000_0000;

4'b1011: out_reg = 16'b0000_1000_0000_0000;

4'b1100: out_reg = 16'b0001_0000_0000_0000;

4'b1101: out_reg = 16'b0010_0000_0000_0000;

4'b1110: out_reg = 16'b0100_0000_0000_0000;

4'b1111: out_reg = 16'b1000_0000_0000_0000;

default: out_reg = 16'b0000_0000_0000_0000;

endcase

end

assign OUT = out_reg;

endmodule

module top_module

(

input [7:0] sw,

input [3:0] btn,

output [7:0] led,

output [6:0] cat,

output [3:0] an,

output [0:0] dp

);

DECODER decoder1

(

.IN(sw[3:0]),

.OUT({dp, cat, led})

);

assign an = 4'b1110;

top_module file design source

Simulation Sources

top_module file simulation source

module top_module_tb();

reg [7:0] SW= 8'b0;

reg [3:0] BTN= 4'b0;

wire [7:0] LED;

wire [6:0] CAT;

wire [3:0] AN;

wire [0:0] DP;

integer i;

top_module uut

(

.sw(SW),

.btn(BTN),

.led(LED),

.cat(CAT),

.an(AN),

.dp(DP)

);

initial

begin

SW[3:0] = 4'b0000; // decoder ve encoder

#10;

SW[3:0] = 4'b0001;

#10;

SW[3:0] = 4'b0010;

#10;

SW[3:0] = 4'b0011;

#10;

SW[3:0] = 4'b0100;

#10;

SW[3:0] = 4'b0101;

#10;

SW[3:0] =4'b0110;

#10;

SW[3:0] =4'b0111;

#10;

SW[3:0] =4'b1000;

#10;

SW[3:0] =4'b1001;

#10;

SW[3:0] =4'b1010;

#10;

SW[3:0] =4'b1011;

#10;

SW[3:0] =4'b1100;

#10;

SW[3:0] =4'b1101;

#10;

SW[3:0] =4'b1110;

#10;

SW[3:0] =4'b1111;

#10;

Simulation Wave

Behavioral simulation wave screenshot

As can be seen from the simulation wave, the design works correctly.  In the "Decoder" truth table, the LED, CAT and AN outputs are given values, respectively, as they should be

RTL Schematic

Technology Schematic

There are 16 LUTs in decoder. There are 7 LUTs for "cat", 1 for "dp" and 8 for "led".  These LUTs are connected to "OUT". "dp" has the most significant bit, "led" has the least significant bit. One bit of "OUT" has a logical value of 1 according to the input statements. These LUTs have values ​​according to their "OUT" statements.

Timing Report

The greatest delay of DECODER is 10.562ns.

PRIORITY ENCODER

| IN[3] | IN[2] | IN[1] | IN[0] | O[1] | O[0] | V |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | X | X | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 0 | 1 | X | 0 | 1 | 1 |
| 0 | 1 | X | X | 1 | 0 | 1 |
| 1 | X | X | X | 1 | 1 | 1 |

Karnaugh Map                                                                     Hand Drawing

Design Sources

module ENCODER

(

input [3:0] IN,

output [1:0] OUT,

output V

);

assign V = IN[0] | IN[1] | IN[2] | IN[3] ;

assign OUT[0] = IN[3] | ( IN[1] & ~(IN[2]) ) ;

assign OUT[1] = IN[3] | IN[2]  ;

endmodule

MSI_Library file design source

top_module file design source

ENCODER encoder1

(

.IN(sw[3:0]),

.OUT(led[1:0]),

.V(led[7])

);

Simulation Sources

Encoder simulation source is same as decoder simulation source.

Simulation Wave

Behavioral simulation wave screenshot

As can be seen from the simulation wave, the design works correctly. The V output (LED[7]) gives the value 1 for all inputs except the 4-bit 0 input. The OUT[0] and OUT[1] outputs, which are connected to LED[0] and LED[1] respectively, are working correctly as the truth table should. Since nothing is connected to the other outputs, they give the Z value.

RTL Schematic

Technology Schematic

RTL schematics represent the project with gates. There are 6 gates in this implementation. However, Technology Schematic represents the project with Fpga sources.  There are 3 LUTs in this implementation. There are not any gates in Fpga. FPGAs are implemented with applications such as LUTs.

Timing Report

The greatest delay of ENCODER is 9.674ns.

Utilization Report

module ENCODER

(

input [3:0] IN,

output [1:0] OUT,

output V

);

reg V_reg;

reg [1:0] OUT_reg;

always@(*)

begin

casex(IN)

4'b0000:

begin

V_reg = 0;

OUT_reg = 2'bxx;

end

4'b0001:

begin

V_reg = 1;

OUT_reg = 2'b00;

end

4'b001x:

begin

V_reg = 1;

OUT_reg = 2'b01;

end

4'b01xx:

begin

V_reg = 1;

OUT_reg = 2'b10;

end

4'b1xxx:

begin

V_reg = 1;

OUT_reg = 2'b11;

end

endcase

end

assign OUT = OUT_reg;

assign V = V_reg;

endmodule

Always- Case Design Code

Timing Report

Utilization Report

Two ENCODER designs have the same source consumption. However, their delay durations are not same. Behavioral design has less delay for "V" out, structural design have less delay for "OUT[0]" output. Output "OUT[1]" has the same delay for both designs.

MULTIPLEXER

Design Sources

MSI_Library file design source

module MUX

(

input [3:0] D,

input [1:0] S,

output O

);

assign O = (S[0] == 1'b0) ?

(S[1] == 1'b0) ? D[0] : D[2]:

(S[1] == 1'b0) ? D[1] : D[3];

endmodule

top_module file design source

MUX  mux1

(

.D(sw[3:0]),

.S(btn[1:0]),

.O(led[0])

);

Simulation Sources

top_module file simulation source

SW[3:0] = 4'b0001; // mux

BTN[1:0] = 2'b00;

#10;

SW[3:0] = 4'b1101;

BTN[1:0] = 2'b01;

#10;

SW[3:0] = 4'b0100;

BTN[1:0] = 2'b010;

#10;

SW[3:0] = 4'b0111;

BTN[1:0] = 2'b11;

#10;

SW[3:0] = 4'b1110;

BTN[1:0] = 2'b00;

#10;

SW[3:0] = 4'b0010;

BTN[1:0] = 2'b01;

#10;

SW[3:0] = 4'b1011;

BTN[1:0] = 2'b010;

#10;

SW[3:0] = 4'b1000;

BTN[1:0] = 2'b11;

#10;

Simulation Wave

Behavioral simulation wave screenshot

As can be seen from the simulation wave, the design works correctly.  The pink glowing part is our output value connected to LED[0]. Our control inputs are connected to the yellow BTN values. When we check the BTN values ​​according to the truth table, the output gives the value entered at the input.

RTL Schematic

Technology Schematic

Timing Report

Utilization Report

Always- Case Design Code

module MUX

(

input [3:0] D,

input [1:0] S,

output O

);

reg O_reg;

always@(*)

begin

case(S)

2'b00 : O_reg = D[0];

2'b01 : O_reg = D[1];

2'b10 : O_reg = D[2];

2'b11 : O_reg = D[3];

endcase

end

assign O = O_reg;

endmodule

Timing Report

Utilization Report

Two MUX designs have the same source consumption. However, there are big differences between the two implementations in terms of the delay of each path. Obviously, always and case blocks have less delay than assign block.

DEMULTIPLEXER

Design Sources

MSI_Library file design source

module DEMUX

(

input D,

input [1:0] S,

output [3:0] O

);

wire and_o_0,and_o_1,and_o_2,and_o_3;

wire s0_not_o,s1_not_o;

NOT s0_not

(

.I(S[0]),

.O(s0_not_o)

);

NOT s1_not

(

.I(S[1]),

.O(s1_not_o)

);

AND and0

(

.l1(s0_not_o),

.l2(s1_not_o),

.O(and_o_0)

);

AND and1

(

.l1(S[0]),

.l2(s1_not_o),

.O(and_o_1)

);

AND and2

(

.l1(s0_not_o),

.l2(S[1]),

.O(and_o_2)

);

AND and3

(

.l1(S[0]),

.l2(S[1]),

.O(and_o_3)

);

TRI tri_0

(

.I(D),

.E(and_o_0),

.O(O[0])

);

TRI tri_1

(

.I(D),

.E(and_o_1),

.O(O[1])

);

TRI tri_2

(

.I(D),

.E(and_o_2),

.O(O[2])

);

TRI tri_3

(

.I(D),

.E(and_o_3),

.O(O[3])

);

endmodule

top_module file design source

DEMUX demux1

(

.D(sw[0]),

.S(btn[1:0]),

.O(led[3:0])

);

Simulation Sources

top_module file simulation source

SW[0] = 1; // demux

BTN[1:0] = 2'b00;

#10;

SW[0] = 1;

BTN[1:0] = 2'b01;

#10;

SW[0] = 1;

BTN[1:0] = 2'b10;

#10;

SW[0] = 1;

BTN[1:0] = 2'b11;

#10;

$finish;

Simulation Wave

Behavioral simulation wave screenshot

As can be seen from the simulation wave, the design works correctly. The value 1 at the top of the image is the input, which is SW[0]. The values ​​in yellow are the BTNs which are control inputs.  BTNs, which are given values ​​according to the truth table values, turn on an LED, which is an output, at each step as it should be. It gives the input value to the LED.  Since no value is given to other LEDs, it takes the Z value. The lit LEDs are pink.

RTL Schematic

Technology Schematic

RESEARCH

- Essentially, latches hold data like flip-flops. However, there are major differences between latches and flip-flops. Flip-flops are edge-triggered components on contrast latches are not edge-triggered. Flip-flops are triggered by the clock signal. Therefore, many flip-flops are controlled simultaneously by the same clock signal. There is an example of D latch in below.

D latches are controlled by enable signal. If the enable signal is logical 1, Q is loaded with the value D. If the enable signal is logical 0, Q is unchanged. Therefore, controlling latches is more difficult than controlling flip-flops.  Due to this asynchronization, timing issues occur.  This timing issue can cause glitches and unnecessary source usage. In addition to these issues, some fpgas may not be able to perform synthesis.

The design should be done with flip-flops. The designer must pay attention to each condition in the if-else and case structure. Assigning else and default condition to if, case structure can resolve the occurrence of the latch.

- Leading zero is the number of zeros from the most significant bit to the least significant bit unless logic 1 is encountered. The leading zero counter is used for floating-point and advanced microprocessors. The circuits are divided into 4-bit priority encoder circuits. The output of each priority encoder is combined. Using priority encoder has advantages for leading zero counter. After most significant logic 1, other bits are do not care. Therefore, there are logic zeros and an logic one for the priority encoder from the most significant bit to the least significant bits. In this way, it is not difficult to calculate logical zeros with the priority encoder.

- Fan-in refers to the maximum input that can be connected to a logic gate. Manufacturers write the fan-in number in their data sheets. Using more inputs than the fan-in numbers will result in additional power consumption and delays.

Fan-out refers to the maximum output that can be connected to a logic gate. Manufacturers write the fan-out number in their data sheets. Using more output than the fan-out numbers will result in additional power consumption and output swings.

Logic gates are made up of transistors. Transistors have parasitic capacitances. These capacitances arise from the nature of the semiconductor structure of the transistors. These parasitic capacitances show capacitive effect. If the user connects more outputs than the fan-out number, these parasitic capacitances cause current drops and swings.

There is no exact value for logic 1 output or logic 0 output. There are three ranges for inverters (the building block of digital electronic circuits). If there are outputs more than fan-out number, it will cause current swings. Therefore, the output signal may drop below the logic 1 range.