DIGITAL SYSTEM DESIGN APPLICATION

EHB436E 	CRN: 11280

Salih Ömer Ongün

040220780

Experiment 4

HALF ADDER

Design Sources

module HA

(

input x,

input y,

output cout,

output sum

);

assign sum = x ^ y ;

assign cout = x & y;

endmodule

Half Adder design source code

Simulation Sources

Half Adder simulation source codes

module HA_tb();

reg X= 1'b0;

reg Y= 1'b0;

wire COUT;

wire SUM;

HA uut

(

.x(X),

.y(Y),

.cout(COUT),

.sum(SUM)

);

initial

begin

X = 0;

Y = 0;

#10;

X = 0;

Y = 1;

#10;

X = 1;

Y = 0;

#10;

X = 1;

Y = 1;

#10;

$finish;

end

endmodule

Simulation Wave

Behavioral simulation wave screenshot

As seen in the simulation, the module works as it should. Sum bit has logic 1 value if one of two inputs has logic 1 value. Cout has logic 1 value if two bit have logic 1 value.

FULL ADDER

Design Sources

Full Adder design source code

module FA

(

input x,

input y,

input cin,

output cout,

output sum

);

wire ha1_sum, ha1_cout, ha2_cout;

HA half1

(

.x(x),

.y(y),

.sum(ha1_sum),

.cout(ha1_cout)

);

HA half2

(

.x(ha1_sum),

.y(cin),

.sum(sum),

.cout(ha2_cout)

);

OR or1

(

.l1(ha1_cout),

.l2(ha2_cout),

.O(cout)

);

endmodule

Simulation Sources

module FA_tb();

reg X= 1'b0;

reg Y= 1'b0;

reg CIN= 1'b0;

wire COUT;

wire SUM;

FA uut

(

.x(X),

.y(Y),

.cin(CIN),

.cout(COUT),

.sum(SUM)

);

initial

begin

X = 0;

Y = 0;

CIN = 0;

#10;

X = 0;

Y = 0;

CIN = 1;

#10;

X = 0;

Y = 1;

CIN = 0;

#10;

X = 0;

Y = 1;

CIN = 1;

#10;

X = 1;

Y = 0;

CIN = 0;

#10;

X = 1;

Y = 0;

CIN = 1;

#10;

X = 1;

Y = 1;

CIN = 0;

#10;

X = 1;

Y = 1;

CIN = 1;

#10;

$finish;

end

endmodule

Full Adder simulation source codes

Simulation Wave

Behavioral simulation wave screenshot

As seen in the simulation, the module works as it should. Sum bit has logic 1 value if one of three (X, Y, CIN) inputs has logic 1 value. Cout has logic 1 if two of three (X, Y, CIN) inputs has logic 1 value. If all inputs have logic 1 values, then both outputs have logic 1 values.

RIPPLE CARRY ADDER

Design Sources

Ripple Carry Adder design source code

module RCA

(

input [3:0] x,

input [3:0] y,

input cin,

output cout,

output [3:0] sum

);

wire cout1,cout2,cout3;

FA fa1

(

.x(x[0]),

.y(y[0]),

.cin(cin),

.sum(sum[0]),

.cout(cout1)

);

FA fa2

(

.x(x[1]),

.y(y[1]),

.cin(cout1),

.sum(sum[1]),

.cout(cout2)

);

FA fa3

(

.x(x[2]),

.y(y[2]),

.cin(cout2),

.sum(sum[2]),

.cout(cout3)

);

FA fa4

(

.x(x[3]),

.y(y[3]),

.cin(cout3),

.sum(sum[3]),

.cout(cout)

);

endmodule

Simulation Sources

Ripple Carry Adder simulation source codes

module RCA_tb();

reg [3:0] X= 4'b0;

reg [3:0] Y= 4'b0;

reg CIN= 1'b0;

wire COUT;

wire [3:0] SUM;

RCA uut

(

.x(X),

.y(Y),

.cin(CIN),

.cout(COUT),

.sum(SUM)

);

initial

begin

X = 4'b0000;

Y = 4'b0000;

CIN = 0;

#10;

X = 4'b0000;

Y = 4'b0000;

CIN = 1;

#10;

X = 4'b0001;

Y = 4'b0010;

CIN = 0;

#10;

X = 4'b0011;

Y = 4'b0011;

CIN = 0;

#10;

X = 4'b0100;

Y = 4'b0100;

CIN = 0;

#10;

X = 4'b1000;

Y = 4'b1000;

CIN = 0;

#10;

X = 4'b1001;

Y = 4'b1011;

CIN = 0;

#10;

X = 4'b1100;

Y = 4'b1010;

CIN = 0;

#10;

X = 4'b1100;

Y = 4'b1010;

CIN = 1;

#10;

X = 4'b1111;

Y = 4'b1111;

CIN = 1;

#10;

$finish;

end

endmodule

Simulation Wave

Behavioral simulation wave screenshot

Ripple Carry Adder has the same working principle as Full Adder. It have three inputs (X, Y, CIN) and two outputs (COUT, SUM). The main difference between the two modules is that Ripple Carry Adder has multiple bit inputs and outputs. Therefore, it uses 4 Full Adder for the module. As seen in the simulation, the module works as it should. If sum of X, Y and CIN have more than 15, COUT has logic 1 values. Because SUM has a 4- bit representation.

PARAMETRIC RIPPLE CARRY ADDER

Design Sources

Parametric Ripple Carry Adder design source code

module parametric_RCA #(parameter SIZE = 8)

(

input [SIZE-1:0] x,

input [SIZE-1:0] y,

input cin,

output cout,

output [SIZE-1:0] sum

);

wire [SIZE:0] cout_gen;

assign cout_gen[0] = cin;

genvar i;

generate

for(i = 0; i<SIZE; i = i+1) begin : gen_full_adder

FA gen_full

(

x[i],

y[i],

cout_gen[i],

cout_gen[i+1],

sum[i]

);

end

endgenerate

assign cout = cout_gen[SIZE];

endmodule

Simulation Sources

Parametric Ripple Carry Adder simulation source codes

module parametric_RCA_tb();

parameter SIZE = 8;

reg [SIZE-1:0] X;

reg [SIZE-1:0] Y;

reg CIN;

wire COUT;

wire [SIZE-1:0] SUM;

parametric_RCA #(.SIZE(SIZE)) uut

(

.x(X),

.y(Y),

.cin(CIN),

.cout(COUT),

.sum(SUM)

);

initial

begin

X = 8'b00000000;

Y = 8'b00000000;

CIN = 0;

#10;

X = 8'b00000000;

Y = 8'b00000000;

CIN = 1;

#10;

X = 8'b00000001;

Y = 8'b00000011;

CIN = 0;

#10;

X = 8'b00001000;

Y = 8'b00001000;

CIN = 0;

#10;

X = 8'b00001100;

Y = 8'b00001011;

CIN = 0;

#10;

X = 8'b00010000;

Y = 8'b00011011;

CIN = 0;

#10;

X = 8'b001001100;

Y = 8'b01001011;

CIN = 0;

#10;

X = 8'b10000000;

Y = 8'b10000000;

CIN = 0;

#10;

X = 8'b01001100;

Y = 8'b00101011;

CIN = 0;

#10;

X = 8'b11111111;

Y = 8'b11111111;

CIN = 0;

#10;

X = 8'b11111111;

Y = 8'b11111111;

CIN = 1;

#10;

$finish;

end

endmodule

Simulation Wave

Behavioral simulation wave screenshot

As seen in the simulation, the module works as it should. The main difference between Ripple Carry Adder and Parametric Ripple Carry Adder is their use of parameters. Therefore, users design any number of bits adder design. SIZE parameter is 8 for the design. If sum of X, Y and CIN have more than 255, COUT has logic 1 values.

RTL Schematic

Technology Schematic

Utilization Report

8 LUT’s is used in the design.

Timing Report

There are maximum delays between the input paths y and output paths count. The greatest delay is between y[0] and the cout path.

PARAMETRIC CARRY LOOKAHEAD ADDER

Design Sources

Parametric Carry Lookahead Adder design source code

module CLA #(parameter SIZE = 8)

(

input [SIZE-1:0] x,

input [SIZE-1:0] y,

input cin,

output cout,

output [SIZE-1:0] s

);

wire [SIZE:0] p;

wire [SIZE:0] g;

wire [SIZE+1:0] c;

assign c[0] = cin;

genvar i;

genvar j;

generate

for(i = 0; i<SIZE; i = i+1) begin: gen_p_g

assign p[i] = x[i] ^ y[i];

assign g[i] = x[i] & y[i];

end

endgenerate

generate

for(j = 0; j<SIZE+1; j = j+1) begin : gen_c

assign c[j+1] = g[j]|(p[j] & c[j]);

end

endgenerate

assign cout = c[SIZE];

generate

for(j = 0; j<SIZE; j = j+1) begin: gen_s

assign s[j] = p[j] ^ c[j];

end

endgenerate

endmodule

Simulation Sources

Parametric Carry Lookahead Adder simulation source codes

module CLA_tb();

parameter SIZE = 8;

reg [SIZE-1:0] X;

reg [SIZE-1:0] Y;

reg CIN;

wire COUT;

wire [SIZE-1:0] S;

CLA #(.SIZE(SIZE)) uut

(

.x(X),

.y(Y),

.cin(CIN),

.cout(COUT),

.s(S)

);

initial

begin

X = 8'b00000000;

Y = 8'b00000000;

CIN = 0;

#10;

X = 8'b00000000;

Y = 8'b00000000;

CIN = 1;

#10;

X = 8'b00000001;

Y = 8'b00000011;

CIN = 0;

#10;

X = 8'b00001000;

Y = 8'b00001000;

CIN = 0;

#10;

X = 8'b00001100;

Y = 8'b00001011;

CIN = 0;

#10;

X = 8'b00010000;

Y = 8'b00011011;

CIN = 0;

#10;

X = 8'b001001100;

Y = 8'b01001011;

CIN = 0;

#10;

X = 8'b10000000;

Y = 8'b10000000;

CIN = 0;

#10;

X = 8'b01001100;

Y = 8'b00101011;

CIN = 0;

#10;

X = 8'b11111111;

Y = 8'b11111111;

CIN = 0;

#10;

X = 8'b11111111;

Y = 8'b11111111;

CIN = 1;

#10;

$finish;

end

endmodule

Simulation Wave

Behavioral simulation wave screenshot

As seen in the simulation, the module works as it should. There are not any differences between parametric ripple carry adder with parametric carry lookahead adder between results. There are differences in the algorithm for these two adders.

RTL Schematic

Technology Schematic

Utilization Report

Timing Report

There are maximum delays between the input paths y and output paths count. The greatest delay is between y[0] and the cout path. Carry Lookahead Adder calculates all results almost simultaneously. On the contrary, Ripple Carry Adder performs parallel computation, for subsequent computations it must wait for the previous computation to finish. There are not any differences for delay time for this code. However, had their input been greater, perhaps we would have achieved different results.

BEHAVIORAL ADDER

Design Sources

Behavioral Adder design source code

module Behav_adder #(parameter SIZE = 8)

(

input [SIZE-1:0] x,

input [SIZE-1:0] y,

output cout,

output [SIZE-1:0] sum

);

assign {cout,sum} = x + y;

endmodule

Simulation Sources

module Behav_adder_tb();

parameter SIZE = 8;

reg [SIZE-1:0] X;

reg [SIZE-1:0] Y;

wire COUT;

wire [SIZE-1:0] SUM;

Behav_adder #(.SIZE(SIZE)) uut

(

.x(X),

.y(Y),

.cout(COUT),

.sum(SUM)

);

initial

begin

X = 8'b00000000;

Y = 8'b00000000;

#10;

X = 8'b00000000;

Y = 8'b00000000;

#10;

X = 8'b00000001;

Y = 8'b00000011;

#10;

X = 8'b00001000;

Y = 8'b00001000;

#10;

X = 8'b00001100;

Y = 8'b00001011;

#10;

X = 8'b00010000;

Y = 8'b00011011;

#10;

X = 8'b001001100;

Y = 8'b01001011;

#10;

X = 8'b10000000;

Y = 8'b10000000;

#10;

X = 8'b01001100;

Y = 8'b00101011;

#10;

X = 8'b11111111;

Y = 8'b11111111;

#10;

X = 8'b11111111;

Y = 8'b11111111;

#10;

$finish;

end

endmodule

Behavioral Adder simulation source codes

Simulation Wave

Behavioral simulation wave screenshot

As seen in the simulation, the module works as it should. There are not any differences between parametric ripple carry adder with behavioral adder between results. There are differences in the algorithm for these two adders.

RTL Schematic

Technology Schematic

Utilization Report

Timing Report

There are different algorithms. Vivado includes these algorithms. If we apply any of these algorithms, Vivado will perform the design with the algorithms we applied. If we use arithmetic operators like add (+) for input, Vivado will choose the most optimal algorithm. The algorithm has optimum delay, power usage and resource usage for our code.  Therefore, Behavioral Adder has the minimum delays in all paths. It uses fewer I/O ports than other algorithms.

DONT_TOUCH Implementation

(* DONT_TOUCH = "yes" *)

module Behav_adder #(parameter SIZE = 8)

(

input [SIZE-1:0] x,

input [SIZE-1:0] y,

output cout,

output [SIZE-1:0] sum

);

assign {cout,sum} = x + y;

endmodule

Design Source Code

Behavioral Adder

There are not any differences in RTL and Technology Schematics between with and without "DONT_TOUCH". They have same delays and resource usage.  A small number of inputs may have resulted in the same results. If we will have greater input, we can see effect of "DONT_TOUCH".

RCA                                            With DONT_TOCUH

Without DONT_TOCUH

There are differences between with and without DONT_TOUCH. Without DONT_TOUCH implementation have less delays and less resource usage.

CLA

Without DONT_TOUCH

With DONT_TOUCH

There aren’t any significant differences in terms of delay for with and without DON’T_TOUCH CLA implementation. However, there are big differences in terms of resource usage. Without the DON’T_TOUCH implementation, much less resources are used.

ADDER – SUBTRACTOR CIRCUIT

Design Sources

module Add_Sub

(

input [3:0] A,

input [3:0] B,

input cin,

output [3:0] sum,

output cout,

output overflow

);

wire [3:0] b_xor;

wire [4:0] c_gen;

assign c_gen[0] = cin;

genvar i;

generate

for(i = 0; i<4; i=i+1) begin: gen_xor

assign b_xor[i] = B[i] ^ cin ;

end

for(i = 0; i<4; i=i+1) begin: gen_full

FA gen_full

(

A[i],

b_xor[i],

c_gen[i],

c_gen[i+1],

sum[i]

);

end

endgenerate

assign cout = c_gen[4];

assign overflow = c_gen[4] ^ c_gen[3];

endmodule

Adder – Subtractor Circuit design source code

Simulation Sources

Adder – Subtractor Circuit simulation source code

module Add_Sub_tb();

reg [3:0] A= 4'b0;

reg [3:0] B= 4'b0;

reg CIN= 1'b0;

wire COUT;

wire [3:0] SUM;

wire OVERFLOW;

Add_Sub uut

(

.A(A),

.B(B),

.cin(CIN),

.cout(COUT),

.overflow(OVERFLOW),

.sum(SUM)

);

integer i;

integer k;

integer j;

initial

begin

for(k = 0; k<2; k = k+1) begin

CIN = k;

for(i = -8; i<8; i = i+1) begin

A = i;

for(j = -8; j<8; j = j+1) begin

B = j;

#5;

end

end

end

$finish;

end

endmodule

RTL Schematic

Technology Schematic

Utilization Report

Timing Report

Simulation Waves

RESEARCH

- DSP Block has different tasks in FPGA. One of them multipliers implementation. Especially, DSP Blocks accelerate and facilitate multiplying. Therefore, multiplication and multiplication related operations are performed in DSP Blocks. If operations cannot be performed in a single DSP Blocks, Vivado use different DSP Blocks or use DSP Blocks and Slice logic together. [1]

Example code for usage DSP Blocks

module DSP_IMP

(

input clk,

input [15:0] a,

input [15:0] b,

input [15:0] c,

output [31:0] result

);

reg [31:0] result_reg;

always @(posedge clk) begin

result_reg <= (a * b) + c;

end

assign result = result_reg;

endmodule

Placement

DSP Blocks accelerate and facilitate multiplying. They use FPGA resources efficiently. They have optimal results in term of power consumption.

- Fixed point representation is a method used to represent fractional numbers. It is used in DSP operations and arithmetic operations.  It has significant advantages compared with other representations for fractional numbers.

It is represented with Qm.n format.

Q: Format of fixed point. M

m : Number of integer bits

n : Number of fractional bits

FRACTIONAL

INTEGER

For Q3.5 format, it have 3 bit integer number, 5 bit fractional number. [2]

| 1 | 0 | 1 |  | 1 | 1 | 0 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

5 is integer part

0.5+0.25+0+0+0.03125=0.78125 is fractional part

101.11001 = 5.78125

Fixed point hardware have less complex than floating points. Therefore, it reduces the size of the chip and results in less power consumption. Generally, fixed-point representation uses fewer bits. Therefore, it requires less memory usage. It may increase the speed of FPGA. The size of the chip and the use of less memory can reduce the price of the FPGA. [3]

References

[1]    DSP Block Implementation • VivaDo Design Suite User Guide: Synthesis (UG901) • Reader • AMD Technical Information Portal. (n.d.). https://docs.amd.com/r/en-US/ug901-vivado-synthesis/DSP-Block-Implementation

[2] Harvie, L. (2024, August 22). How to perform Fixed-Point Arithmetic on an FPGA. RunTime Recruitment. https://runtimerec.com/how-to-perform-fixed-point-arithmetic-on-an-fpga/

[3] Benefits of Fixed-Point Hardware. (n.d.). https://www.mathworks.com/help/fixedpoint/gs/benefits-of-fixed-point-hardware.html