DIGITAL SYSTEM DESIGN APPLICATION

EHB436E 	CRN: 11280

Salih Ömer Ongün

040220780

Experiment 6

CLOCK GENERATION

Design Source

module clock_gen

(

input clk,

input rst,

output [6:0] cnt100,

output [6:0] cnt80,

output [6:0] cnt60

);

wire clk80;

wire clk60;

reg [6:0] cnt100_reg;

reg [6:0] cnt80_reg;

reg [6:0] cnt60_reg;

clk_wiz_0 instance_name

(

// Clock out ports

.clk_out1(clk80),     // output clk_out1 80MHz

.clk_out2(clk60),     // output clk_out2 60MHz

// Status and control signals

.reset(rst), // input reset

.locked(locked),       // output locked

// Clock in ports

.clk_in1(clk)      // input clk_in1

);

always @(posedge clk) begin

if(rst==1'b1) begin

cnt100_reg <= 7'b0;

end

else if(locked ==1'b1) begin

if(cnt100_reg < 100) begin

cnt100_reg <= cnt100_reg + 1;

end

else begin

cnt100_reg <= cnt100_reg;

end

end

end

assign cnt100 = cnt100_reg;

always @(posedge clk80) begin

if(rst==1'b1) begin

cnt80_reg <= 7'b0;

end

else if(locked ==1'b1) begin

if(cnt80_reg < 80) begin

cnt80_reg <= cnt80_reg + 1;

end

else begin

cnt80_reg <= cnt80_reg;

end

end

end

assign cnt80 = cnt80_reg;

always @(posedge clk60) begin

if(rst==1'b1) begin

cnt60_reg <= 7'b0;

end

else if(locked ==1'b1) begin

if(cnt60_reg < 60) begin

cnt60_reg <= cnt60_reg + 1;

end

else begin

cnt60_reg <= cnt60_reg;

end

end

end

assign cnt60 = cnt60_reg;

endmodule

module clock_gen_tb();

reg CLK = 1'b0;

reg RST = 1'b0;

wire [6:0] CNT100;

wire [6:0] CNT80;

wire [6:0] CNT60;

clock_gen uut

(

.clk(CLK),

.rst(RST),

.cnt100(CNT100),

.cnt80(CNT80),

.cnt60(CNT60)

);

always begin

#5 CLK = ~CLK;

end

initial begin

RST = 1;

#50;

RST = 0;

#2000;

$finish;

end

endmodule

Simulation Source

Simulation Wave

As can be seen in the image, our code is working correctly. The cnt100 counter runs on the rising edge of the clk clock. The cnt60 counter runs on the rising edge of the clk60 clock. The cnt80 counter runs on the rising edge of the clk80 clock. When cnt100 reaches 100, the counter stops. When cnt60 reaches 60, the counter stops. When cnt80 reaches 80, the counter stops.

RTL Schematic

Technology Schematic

Utilization Report

Clock Network Report

We use 100MHz, 80MHz and 60MHz clocks.

CLOCK GATING

Design Source

module clock_gating

(

input clk,

input rst,

output [6:0] cnt50,

output [6:0] cnt25

);

reg clk50_en,clk25_en;

wire clk50,clk25;

reg [6:0] cnt50_reg;

reg [6:0] cnt25_reg;

BUFR #(                 // 50MHz

.BUFR_DIVIDE("2"),

.SIM_DEVICE("7SERIES")

)

BUFR_inst50 (

.O(clk50),

.CE(clk50_en),

.CLR(1'b0),

.I(clk)

);

BUFR #(                 // 25MHz

.BUFR_DIVIDE("4"),

.SIM_DEVICE("7SERIES")

)

BUFR_inst25 (

.O(clk25),

.CE(clk25_en),    // 1-bit input: Active high, clock enable (Divided modes only)

.CLR(1'b0), // 1-bit input: Active high, asynchronous clear (Divided modes only)

.I(clk)

);

always @(posedge clk50, posedge rst) begin

if(rst==1'b1) begin

clk50_en <= 1'b1;

cnt50_reg <= 7'b0;

end

else if(clk50_en == 1'b1)begin

if(cnt50_reg < 50) begin

cnt50_reg <= cnt50_reg + 1;

end

else begin

clk50_en <= 1'b0;

end

end

end

assign cnt50 = cnt50_reg;

always @(posedge clk25, posedge rst) begin

if(rst==1'b1) begin

clk25_en <= 1'b1;

cnt25_reg <= 7'b0;

end

else if(clk25_en == 1'b1)begin

if(cnt25_reg < 25) begin

cnt25_reg <= cnt25_reg + 1;

end

else begin

clk25_en <= 1'b0;

end

end

end

assign cnt25 = cnt25_reg;

endmodule

module clock_gating_tb();

reg CLK = 1'b0;

reg RST = 1'b0;

wire [6:0] CNT50;

wire [6:0] CNT25;

clock_gating uut

(

.clk(CLK),

.rst(RST),

.cnt50(CNT50),

.cnt25(CNT25)

);

always begin

#5 CLK = ~CLK;

end

initial begin

RST = 1;

#105;

RST = 0;

#800;

RST = 1;

#105;

RST = 0;

$finish;

end

endmodule

Simulation Source

Simulation Wave

As can be seen in the image, our code is working correctly. Our internal 100MHz clock is available in clk. External clk50(50MHz) and clk25(25MHz) work correctly. The cnt50 counter runs on the rising edge of the clk50 clock. The cnt25 counter runs on the rising edge of the clk25 clock. If we give the rst signal the counters and enable signals are reset. However, the clocks continue to run.

After the rst signal, counters start counting.

If counters reach their final value, enable signal of clocks have logic zero value. Therefore, counters stop counting and clocks stop.

Technology Schematic

Utilization Reports

Clock Networks

We use 50MHz and 25MHz clocks.

BLOCK RAM

Design Source

module block_ram

(

input clka,

input wea,

input [3:0] addra,

input [7:0] dina,

output [7:0] douta

);

wire clk_50;

blk_mem_gen_0 your_instance_name

(

.clka(clk_50),    // input wire clka

.wea(wea),      // input wire [0 : 0] wea

.addra(addra),  // input wire [3 : 0] addra

.dina(dina),    // input wire [7 : 0] dina

.douta(douta)  // output wire [7 : 0] douta

);

clk_wiz_2 instance_name

(

// Clock out ports

.clk_50(clk_50),     // output clk_50

// Status and control signals

.reset(reset), // input reset

.locked(locked),       // output locked

// Clock in ports

.clk_in1(clka)      // input clk_in1

);

endmodule

module block_ram_tb();

reg clka = 1'b0;

reg wea = 1'b0;

//reg ena = 1'b1;

reg [3:0] addra = 4'b0;

reg [7:0] dina = 8'b0;

wire [7:0] douta;

integer i;

reg [7:0] vary [15:0];

//reg [7:0] vary;

//reg [7:0] vary2;

block_ram uut

(

.clka(clka),

.wea(wea),

.addra(addra),

.dina(dina),

.douta(douta)

);

always begin

#5 clka = ~clka;

end

initial begin

$display("we are reading");

#300; // ilk baslangcta LOCKED oluyor

for(i = 0; i<16; i=i+1) begin

addra = i;

#50;

$display("Address: %d, Data Read: %h", addra, douta);

end

#10

$display("we are writing to array");

for(i = 0; i<16; i=i+1) begin

addra = i;

#50;

vary[15-i] = douta;

$display("array'e yaz  Address: %d, Data Output: %h,Vary: %h", addra, douta,vary[15-i]);

end

#10;

$display("we are writing to address");

wea = 1'b1;

for(i = 0; i<16; i=i+1) begin

addra = i;

dina = vary[i];

#50

$display("Address: %d, Data Input: %h, Vary: %h, Data Output: %h", addra, dina,vary[i],douta);

end

$display("we are reading number backwards");

wea = 1'b0;

for(i = 0; i<16; i=i+1) begin

addra = i;

#50;

$display("Address: %d, Data Read: %h", addra, douta);

end

$finish;

end

endmodule

Simulation Source

Simulation TCL Console

First, we read the data filled from the memory.coe file. It read my numbers in the correct order. Then, I created an array and created the data that the block ram wrote to the array in reverse order. For example, the last index of the array has the first index data of the block ram.

Then the data in the array is written to the block ram address in order. My ID number has been written to block ram in reverse order. Finally, ıt read value in block ram, the values is inverse order of my student ID.

Technology Schematic

Utilization Reports

FIFO

module FIFO

(

input clk,

input rst,

input wr_en,

input rd_en,

input [7:0] din,

output [7:0] dout,

output empty,

output full,

output overflow,

output underflow

);

wire clk_50,clk_25;

fifo_generator_0 your_instance_name

(

.wr_clk(clk_50),        // input wire wr_clk

.wr_rst(rst),        // input wire wr_rst

.rd_clk(clk_25),        // input wire rd_clk

.rd_rst(rst),        // input wire rd_rst

.din(din),              // input wire [7 : 0] din

.wr_en(wr_en),          // input wire wr_en

.rd_en(rd_en),          // input wire rd_en

.dout(dout),            // output wire [7 : 0] dout

.full(full),            // output wire full

.overflow(overflow),    // output wire overflow

.empty(empty),          // output wire empty

.underflow(underflow)  // output wire underflow

);

clk_wiz_3 instance_name

(

// Clock out ports

.clk_50(clk_50),     // output clk_50

.clk_25(clk_25),     // output clk_25

// Status and control signals

.reset(reset), // input reset

.locked(locked),       // output locked

// Clock in ports

.clk_in1(clk)      // input clk_in1

);

endmodule

Design Source

module FIFO_tb();

reg clk = 1'b0;

reg rst = 1'b0;

reg wr_en = 1'b0;

reg rd_en = 1'b0;

reg [7:0] din = 1'b0;

wire [7:0] dout;

wire empty;

wire full;

wire overflow;

wire underflow;

integer i = 0;

FIFO uut

(

.clk(clk),

.rst(rst),

.wr_en(wr_en),

.rd_en(rd_en),

.din(din),

.dout(dout),

.empty(empty),

.full(full),

.overflow(overflow),

.underflow(underflow)

);

always begin

#5 clk = ~clk;

end

initial begin

#200;

rst = 1'b1;

#20;

rst = 1'b0;

#350;

while(i<65) begin

wr_en = 1'b1;

din = i;

$display("Data Write: %d",din);

rd_en = 1'b1;

i=i+1;

#10;

$display("Data Read: %d",dout);

end

end

endmodule

Simulation Source

Simulation Wave

Simulation start to write data.

Write clock has 50MHz and read clock 25MHz frequency. Therefore, writing is faster than reading. Writing reach final value but reading does not.

Writing reach final value but reading does not. Therefore, full flag has logic 1 value. Simulation write data to FIFO although full has logic 1 value. Therefore, overflow flag has logic 1 value.

Finally, reading reach final value but it can not read some intermediate value.

RTL Schematic

Utilization Report

Clock Network Reports

We use 50MHz and 25MHz clocks.

Clock Network Interaction

Post-Implementation Timing Simulation

RESEARCH

- Clock Tree Synthesis is one of most important technique in digital desgin. It aims distribute clock signal balanced. It insert buffers/inverters along the clock routes of an ASIC design. [1]

Clock Latency : Clock latency refers to the arrival time of the clock signal from the clock source to flip flops. The clock latency from the port up to the clock pin is referred to as the network latency.

Clock Skew : Clock Skew refers to the difference in the clock arrival time between two registers.

Clock Slew (or transition time): The time that a given signal takes to rise from a level of 10% of the rail voltage to the level of 90% of the rail voltage is referred to as rise slew. Similarly, the time that a given signal takes to fall from a level of 90% of the rail voltage to the level of 10% of the rail voltage is referred to as fall slew.[2]

Clock Jitter: Normally, clocks have specific period. However, noise, temperature and other effects cause disruption in period of clock cycle. It is called clock jitter.

Conventional CTS/Single point CTS: 
Single point CTS is the default choice for most of the designers having lower frequency & lesser no of sinks. As name suggested having single clock source which distribute clock to every corner of design.

Clock Mesh Structure
As the name suggests it create a dense mesh of shorted wires which is being driven by mesh drivers to distribute clock in every corner of the design.
The output of all the mesh drivers will be shorted using a metal mesh, which will carry the clock signal across the block using horizontal and vertical metal stripes

[3]

- If the signal is not in a stable state at a certain time before and after the clock signal, the problem of metastability occurs. One of the methods to solve metastability is the Double Flop Synchroniser method. This method consists of two cascaded D flip-flops that sample the input signal with the destination clock. Flip-flops work as buffers to solve the metastability problem. Signals that cannot reach a stable state in the first flip-flop become stable due to the delay that occurs until they reach the second flip-flop. Therefore, it can solve metastability problem.  [4]

[5]

REFERENCE

[1] B, D. M. (2024, March 27). What is Clock Tree Synthesis? ChipEdge VLSI Training Company. https://chipedge.com/what-is-clock-tree-synthesis/

[2] Anysilicon. (2022, September 24). Ultimate Guide: Clock Tree Synthesis - AnySilicon. AnySilicon. https://anysilicon.com/clock-tree-synthesis/

[3] Abhishek. (n.d.). Different types of clock tree structure. https://vlsiconceptsforyou.blogspot.com/2020/07/different-types-of-clock-tree-structure.html?m=1

[4] Satheesh, M. (2024, April 3). Double Flip-Flop Synchronizer for CDC. Digital System Design. https://digitalsystemdesign.in/double-flip-flop-synchronizer-for-cdc/?srsltid=AfmBOooDFmO3E6dyGGnOqOVNQIvss2JmlYAyPtwuCR7QY3uE-IxriopT

[5] File:2FF Synchronizer.gif - Wikimedia Commons. (2020, January 18). https://commons.wikimedia.org/wiki/File:2FF_synchronizer.gif