DIGITAL SYSTEM DESIGN APPLICATION

EHB436E 	CRN: 11280

Salih Ömer Ongün

040220780

Experiment 7

Encoding Methods

In Binary encoding, numbers are coded in ascending binary order. Therefore, it is simply for understanding. It uses more than one flip flop simultaneously so that it use less flip-flop for all coding. It is durable against to noise and it have advantages in terms of memory usage.

| State | Binary Encoding |
| --- | --- |
| State A | 000 |
| State B | 001 |
| State C | 010 |

In Gray coding, bit changes occur during state transitions. It is used in Karnaugh maps. It has 00,01,11,10 encoding in Karnaugh maps. It has different advantages. It changes one bit. Therefore, it has less power consumption. Gary encoding uses protection for asynchronous outputs from glitches. It is another advantage of Gray encoding.

| State | Gray Encoding |
| --- | --- |
| State A | 000 |
| State B | 001 |
| State C | 011 |

In One-Hot encoding, states have one logic 1 bit. Other bits are logic 0. Logic 1 bit is shifted left during state transition. One-hot encoding makes these combinational circuits simpler, which reduces propagation delay, which in turn makes the FSM compatible with higher clock frequencies. One-hot encoding increases the number of FFs and it increase memory usage.[1]

| State | One- Hot Encoding |
| --- | --- |
| State A | 001 |
| State B | 010 |
| State C | 100 |

CONSECUTIVE 1 OR 0 MODEL

Design Source

Mealy machine type code

module consec

(

input clk,

input rst,

input x,

output z

);

reg q0,q1,q2;

wire Q0,Q1,Q2;

reg z_reg;

assign Q2 = (x & q2) | (x & q1 & q0);

assign Q1 = (q1 & ~q0) | (x & ~q2 & ~q1) | (~q2 & ~q1 & q0);

assign Q0 = (x & ~q1) | (~q0 & ~q1 & ~q2) | (x & ~q0);

assign z = (x & q2 & q0) | (~x & q1 & ~q0);

always @(posedge clk) begin

if(rst == 1'b1) begin

q2 <= 1'b0;

q1 <= 1'b0;

q0 <= 1'b0;

end

else begin

q2<= Q2;

q1<= Q1;

q0<= Q0;

end

end

endmodule

Simulation Source

module consec_tb();

reg clk = 1'b0;

reg rst = 1'b1;

reg x = 1'b1;

wire z;

reg [41:0] test = 42'b010011000111000011110000011111000000111111;

integer i= 41;

consec uut

(

.clk(clk),

.rst(rst),

.x(x),

.z(z)

);

always begin

#5 clk = ~clk;

end

initial begin

#10;

rst = 1'b0;

for(i = 41; i>=0; i = i-1) begin

x = test[i];

#10;

end

$finish;

end

endmodule

Simulation Waveform

z obtain logic 1 value after three or more consecutive 1s or 0s. It should obtain logic 1 after four consecutive 1s or 0s. It has faulty 0 and faulty 1 values in three consecutive 1s and 0s.

module consec

(

input clk,

input rst,

input x,

output z

);

reg q0,q1,q2;

wire Q0,Q1,Q2,z_moore;

reg z_reg;

assign Q2 = (x & q2) | (x & q1 & q0);

assign Q1 = (q1 & ~q0) | (x & ~q2 & ~q1) | (~q2 & ~q1 & q0);

assign Q0 = (x & ~q1) | (~q0 & ~q1 & ~q2) | (x & ~q0);

assign z_moore = (x & q2 & q0) | (~x & q1 & ~q0);

always @(posedge clk) begin

if(rst == 1'b1) begin

q2 <= 1'b0;

q1 <= 1'b0;

q0 <= 1'b0;

end

else begin

q2<= Q2;

q1<= Q1;

q0<= Q0;

z_reg<= z_moore;

end

end

assign z = z_reg;

endmodule

Moore machine type code

Moore type machine gives correct results. z output obtains logic 1 value after four consecutive 1s or 0s.

Post -Implementation Timing Simulation

Post -Implementation Timing Simulation have same results with Moore type simulation.

Utilization Report

Timing Report

Undesirable States

I give 110 state for initialization. Circuit does not stuck any state. It just changes the order of events.

Behavioral Model

Post -Implementation Timing Simulation

It gives correct results like previous designs.

Utilization Report

Timing Report

Behavioral model and my previous model have the same resource usage. However, behavioral model has less delay results than previous design. Max delay of previous design is 6.044 ns but max delay of behavioral design is 4.763ns. There is a small difference in terms of delays.

DIVIDED MODEL

Design Source

module divide

(

input clk,

input rst,

input x,

output z

);

reg q0,q1,x_cs,z_reg;

wire Q0,Q1,a;

assign Q1 = (a & q1) | (a & q0) ;

assign Q0 = (a & ~q1 & ~q0);

assign a = ~(x_cs ^ x);

assign z = (a & q1);

always @(posedge clk) begin

if(rst == 1'b1) begin

q1 <= 1'b0;

q0 <= 1'b0;

end

else begin

q1 <= Q1;

q0 <= Q0;

x_cs<= x;

end

end

endmodule

Mealy type code

Simulation Source

module divide_tb();

reg clk = 1'b0;

reg rst = 1'b1;

reg x;

wire z;

reg [41:0] test = 42'b010011000111000011110000011111000000111111;

integer i= 41;

divide uut

(

.clk(clk),

.rst(rst),

.x(x),

.z(z)

);

always begin

#5 clk = ~clk;

end

initial begin

#10;

rst = 1'b0;

for(i = 41; i>=0; i = i-1) begin

x = test[i];

#10;

end

$finish;

end

endmodule

Simulation Waveform

z obtain logic 1 value after three or more consecutive 1s or 0s. It should obtain logic 1 after four consecutive 1s or 0s. It has faulty 0 and faulty 1 values in three consecutive 1s and 0s.

module divide

(

input clk,

input rst,

input x,

output z

);

reg q0,q1,x_cs,z_reg;

wire Q0,Q1,a,z_moore;

assign Q1 = (a & q1) | (a & q0) ;

assign Q0 = (a & ~q1 & ~q0);

assign a = ~(x_cs ^ x);

assign z_moore = (a & q1);

always @(posedge clk) begin

if(rst == 1'b1) begin

q1 <= 1'b0;

q0 <= 1'b0;

end

else begin

q1 <= Q1;

q0 <= Q0;

x_cs<= x;

z_reg<= z_moore;

end

end

assign z = z_reg;

endmodule

Moore type code

Moore type machine gives correct results. z output obtains logic 1 value after four consecutive 1s or 0s.

Post -Implementation Timing Simulation

Post -Implementation Timing Simulation have same results with Moore type simulation.

Utilization Report

Timing Report

Behavioral Model

Design Source

module div_behav

(

input clk,

input rst,

input x,

output z

);

reg z_reg,a,x_cs;

localparam A = 2'b00;

localparam B = 2'b01;

localparam C = 2'b10;

reg [1:0] state;

always@(*) begin

a<= ~(x_cs ^ x);

end

always @(posedge clk, posedge rst) begin

if(rst == 1) begin

state <= A;

a<=0;

x_cs<=0;

end

else begin

x_cs<=x;

case(state)

A : begin

if(a==1) begin

state <= B;

end

else begin

state <= A;

end

z_reg<= 0;

end

B: begin

if(a==1) begin

state<= C;

end

else begin

state<= A;

end

z_reg<=0;

end

C: begin

if(a==1) begin

state<= C;

z_reg<= 1;

end

else begin

state<= A;

z_reg<=0;

end

end

default: begin

state <= A;

z_reg<= 0;

end

endcase

end

end

assign z = z_reg;

endmodule

module div_behav_tb();

reg clk = 1'b0;

reg rst = 1'b1;

reg x;

wire z;

reg [41:0] test = 42'b010011000111000011110000011111000000111111;

integer i= 41;

div_behav uut

(

.clk(clk),

.rst(rst),

.x(x),

.z(z)

);

always begin

#5 clk = ~clk;

end

initial begin

#10

rst = 1'b0;

for(i = 41; i>=0; i = i-1) begin

x = test[i];

#10;

end

$finish;

end

endmodule

Simulation Source

Simulation Waveform

Behavioral model gives correct results. z output obtains logic 1 value after four consecutive 1s or 0s.

Post -Implementation Timing Simulation

Utilization Report

Timing Report

Behavioral model and my previous model have the same resource usage. Behavioral model and my previous model have same delays.

REFERENCES

[1] Arar, S. (2018, March 5). Encoding the states of a finite state machine in VHDL. Technical Articles. https://www.allaboutcircuits.com/technical-articles/encoding-the-states-of-a-finite-state-machine-vhdl/