DIGITAL SYSTEM DESIGN APPLICATION

EHB436E 	CRN: 11280

PROJECT2

Salih Ömer Ongün

040220780

UART PROTOCOL

Communication protocols are very important in the use of electronic integrated circuits and devices. Certain rules must be followed for two or more electronic components to be understood and communicated correctly. For this reason, there are many protocols in communication systems. The UART protocol is one of them.

UART uses two wires to transmit and receive data. UART is a hardware communication protocol that uses asynchronous serial communication with configurable speed. Asynchronous means there is no clock signal to synchronize the output bits from the transmitting device going to the receiving end.[1]

[1]

The clk signal is not used for synchronization of data in UART. Data is sent asynchronously. There is no data transmission in UART based on the clk signal, there is asynchronous transmission. Tx and rx process the data they obtain according to the specified baud rate. There are different baud rates. It is necessary to decide in advance which rate to use.

[1]

Normally, the logic 1 bit is always on. When the start bit becomes logic zero, the system starts working. It usually has 8 bits of data. In addition, the stop bit is sent at the end to stop the system. The stop bit is  logic 1.

[2]

The transmitter receives data in parallel. It samples within itself with the baud rate frequency and gives parallel data to the output serially. The receiver receives the data serially. It samples the data with the baud rate frequency determined within itself and gives the serial data to the output in parallel.

[1]

ALGORITHM

I create four modules. These are uart_rx, uart_tx, baud_gen, uart_top. The uart_rx module acts as the receiver of the uart. The uart_tx module acts as the transmitter of the uart. It has a baud gen frequency divider structure. Therefore, it produces the baud rate we need and 8x equivalent of the baud rate. Uart_top is the top module where I combine all the modules.

Baud Rate Generator Module

This module produces the desired baud rate and 8x the equivalent of the baud rate, thanks to its parameterized structure from the internal 100 MHz clk signal. For this project, the baud rate should be 9600 and 115200.

module baud_gen #(parameter [16:0]  freq= 115200)

(

input clk,

input rst,

input baud_en,

output count_8x_ready,

output count_baud_ready

);

localparam integer clk_freq = 100000000; // Internal clock frequency (100 MHz)

reg count_8x_ready_reg,count_baud_ready_reg;

reg [13:0] counter_8x;

reg [2:0] counter_baud;

always @(posedge clk) begin

if(rst==1'b1) begin

counter_baud <= 3'b0; // baud counter

count_baud_ready_reg <= 0; // baud counter result

counter_8x <= 13'b0; // 8x counter

count_8x_ready_reg <= 0; // 8x counter result

end

else if (baud_en == 1) begin

if(counter_8x ==((clk_freq/(freq*8))-1)) begin

counter_8x <= 13'b0;

count_8x_ready_reg <= 1;

if(counter_baud == 7) begin

counter_baud <= 3'b0;

count_baud_ready_reg <= 1;

end

else begin

counter_baud <= counter_baud + 1;

count_baud_ready_reg <= 0;

end

end

else begin

counter_8x <= counter_8x + 1;

count_8x_ready_reg <= 0;

count_baud_ready_reg <= 0;

end

end

else begin

count_8x_ready_reg <= 0;

counter_8x <= 13'b0;

counter_baud <= 13'b0;

count_baud_ready_reg <= 0;

end

end

assign count_8x_ready = count_8x_ready_reg;

assign count_baud_ready = count_baud_ready_reg;

endmodule

Design Source

First, I defined our internal 100 MHz frequency for frequency division in the module. Then I created counters for both 8x and baud rate. Since 8x will have higher frequency and lower period, I calculated it first and gave it to the output. Then, when 8x becomes logical 1 every 8 times, I obtained the desired baud rate. For the operations to take place, that is, for the frequency division operation to take place, baud_en must be 1. If it becomes zero and then becomes one again, the counting process starts over. I used this in the receiver.

Simulation Source

module baud_gen_tb();

// Parameters

parameter [16:0] freq = 115200;

// Testbench Signals

reg clk = 1'b0;    // Clock

reg rst = 1'b0;    // Reset

reg baud_en = 1'b0; // Enable signal

wire count_8x_ready; // 8x clock ready

wire count_baud_ready; // Baud clock ready

// Instantiate the DUT (Device Under Test)

baud_gen #(.freq(freq)) uut (

.clk(clk),

.rst(rst),

.baud_en(baud_en),

.count_8x_ready(count_8x_ready),

.count_baud_ready(count_baud_ready)

);

// Generate a 100 MHz clock (period = 10 ns)

always begin

#5 clk = ~clk;

end

initial begin

rst = 1;

#10;

rst = 0;

baud_en = 1;

#11000;

baud_en = 0;

#200;

$finish;

end

endmodule

Simulation Waveform

baud_div=108freq-1

First, I will simulate the baud rate for 115200 Hz frequency.

As you can see in the picture, the code is working correctly.

baud_div=108115200*8-1=107

Second,I will simulate the baud rate for 9600 Hz frequency.

baud_div=1089600*8-1=1301

As you can see in the picture, the code is working correctly.

UART TRANSMITTER

DESIGN SOURCE

module uart_tx #(parameter [16:0]  freq= 115200)

(

input clk,

input rst,

input [7:0] d_in,

input tx_en,

output seri_out,

output reg start,

output reg busy,

output done

);

wire count_baud_ready,out_of_use;

reg done_reg, seri_out_reg,baud_en;

reg [2:0] bit_in;

reg [7:0] buffer;

localparam IDLE = 2'b00;

localparam START = 2'b01;

localparam DATA = 2'b10;

localparam STOP = 2'b11;

reg [1:0] state;

baud_gen #(.freq(freq)) divider

(

.clk(clk),

.rst(rst),

.baud_en(baud_en),

.count_8x_ready(out_of_use),

.count_baud_ready(count_baud_ready)

);

always @(posedge clk) begin

if (rst == 1'b1) begin

state <= IDLE;

done_reg <= 0;

start <= 0;

busy <= 0;

seri_out_reg <= 1;

baud_en <= 0;

end

else begin

case(state)

IDLE: begin

done_reg <= 0;

seri_out_reg <= 1;

baud_en <= 0;

buffer <= d_in;

if (tx_en == 1) begin

state <= START;

bit_in <= 0;

end

end

START: begin

start <= 1;

seri_out_reg <= 0;

baud_en <= 1;

if(count_baud_ready == 1) begin

state <= DATA;

end

end

DATA: begin

start <= 0;

busy <=1;

seri_out_reg <= buffer[0];

if(count_baud_ready == 1) begin

if(bit_in == 7) begin

state <= STOP;

bit_in <= 0;

end

else begin

buffer[6:0] <= buffer[7:1];

bit_in <= bit_in + 1;.

end

end

end

STOP: begin

seri_out_reg <= 1;

if(count_baud_ready ==1) begin

state <= IDLE;

baud_en <= 0;

busy <=0;

done_reg <= 1;

end

end

default: begin

state <= IDLE;

done_reg <= 0;

seri_out_reg <= 1;

baud_en <= 0;

end

endcase

end

end

assign done = done_reg;

assign seri_out = seri_out_reg;

endmodule

Since I made the transmitter before writing the top module, I had to instantiate the baud_gen module in the transmitter. When I made the top module, I connected them all in the top module.

Our transmitter normally remains in IDLE state. If we make each enable signal logical 1, the system starts working and goes to the START state.

In the START state, our output becomes logical 0 to create the start bit during 1 baud frequency. Then it goes to DATA state. In DATA, first the first value of the input is given to the output and waits for 1 baud frequency. Then, with the expression “buffer[6:0] <= buffer[7:1];”, I shift the input that I assigned to the buffer one bit to the right and give it to the output. It also waits for 1 baud frequency. This process goes until the last bit of the input. Then it goes to STOP state.

In the stop state, the stop bit, logical 1, is given to the output during 1 baud frequency. Finally, the done output becomes 1 and the process ends.

Finite State Machine Transmitter

Simulation Source

module uart_tx_tb;

parameter [16:0] freq = 115200;

reg clk = 1'b0;

reg rst = 1'b0;

reg [7:0] d_in;

reg tx_en = 1'b0;

wire seri_out;

wire done;

integer file, r;

reg [7:0] stimulus_data;

integer i;

uart_tx #(.freq(freq)) uut (

.clk(clk),

.rst(rst),

.d_in(d_in),

.tx_en(tx_en),

.seri_out(seri_out),

.done(done)

);

// Saat sinyali üretimi

always #5 clk = ~clk; // 10ns periodlu (100 MHz)

initial begin

file = $fopen("stimulus2.txt", "r");

if (file == 0) begin

$display("Error: Cannot open stimulus2.txt");

$finish;

end

rst = 1;

d_in = 8'b0;

tx_en = 0;

#8000;

rst = 0;

for (i = 0; i < 4; i = i + 1) begin

r = $fscanf(file, "%b\n", stimulus_data);

if (r == 1) begin

d_in = stimulus_data;

tx_en = 1;

#10;

tx_en = 0;

wait (done == 1);

#15000;

end

end

$fclose(file);

$finish;

end

endmodule

I wrote python code to generate random numbers. In the simulation, I took 4 data from the file I created in python and checked it.

import random

def gen_binary():

with open("stimulus2.txt","w") as f:

for k in range(0,4):

number = random.getrandbits(8)

bin_num = format(number, "0b")

print(len(bin_num))

if len(bin_num)<8:

for i in range(0,8-len(bin_num)):

bin_num = "0" + str(bin_num)

f.write(bin_num +"\n")

print("----------------")

Phyton Code

10110100

01101110

11011000

10110011

File Content

Simulation Waveform

As seen in the picture, the code works correctly. I took 4 different 8-bit random numbers from the file. I tested it and serial_out gave the correct output.

UART RECEIVER

DESIGN SOURCE

module  uart_rx #(parameter [16:0]  freq= 115200)

(

input clk,

input rst,

input d_in,

input rx_en,

output [7:0] d_out,

output reg start,

output reg busy,

output done

);

reg [3:0] count_ones;

reg [3:0] count_zeros;

reg [2:0] counter_halfperiod;

wire count_baud_ready,count_8x_ready;

reg done_reg,baud_en;

reg [7:0] d_out_reg;

reg [3:0] bit_in;

localparam IDLE = 2'b00;

localparam START = 2'b01;

localparam DATA = 2'b10;

localparam STOP = 2'b11;

reg [1:0] state;

baud_gen #(.freq(freq)) divider

(

.clk(clk),

.rst(rst),

.baud_en(baud_en),

.count_8x_ready(count_8x_ready),

.count_baud_ready(count_baud_ready)

);

always @(posedge clk) begin

if (rst == 1'b1) begin

state <= IDLE;

done_reg <= 0;

start <= 0;

busy <= 0;

baud_en <= 0;

counter_halfperiod <= 2'b0;

count_ones <= 0;

count_zeros <= 0;

end

else begin

case(state)

IDLE: begin

done_reg <= 0;

baud_en <= 0;

if (rx_en == 1) begin

state <= START;

bit_in <= 0;

start <=1;

end

end

START: begin

if(d_in == 0) begin

baud_en <= 1;

if(counter_halfperiod < 4) begin

if(count_8x_ready == 1) begin

counter_halfperiod = counter_halfperiod + 1;

end

end

else begin

state <= DATA;

busy <=1;

start <= 0;

counter_halfperiod = 2'b0;

baud_en <= 0;

end

end

end

DATA: begin

baud_en <= 1;

if(count_baud_ready == 1) begin

if(count_ones > count_zeros) begin

d_out_reg = {1'b1,d_out_reg[7:1]};

end

if(count_ones < count_zeros) begin

d_out_reg = {1'b0,d_out_reg[7:1]};

end

if(bit_in == 7) begin

busy <= 0;

state <= STOP;

bit_in <= 0;

end

else begin

bit_in <= bit_in + 1;

end

count_ones = 3'b0;

count_zeros = 3'b0;

end

else begin

if(count_8x_ready == 1) begin

if(d_in == 1) begin

count_ones = count_ones + 1;

end

else begin

count_zeros = count_zeros + 1;

end

end

end

end

STOP: begin

if(count_baud_ready == 1) begin

state <= IDLE;

baud_en <= 0;

done_reg <= 1;

end

end

default: begin

state <= IDLE;

done_reg <= 0;

d_out_reg <= 1;

baud_en <= 0;

end

endcase

end

end

assign done = done_reg;

assign d_out = d_out_reg;

endmodule

Since I made the receiver before writing the top module, I had to instantiate the baud_gen module in the receiver. When I made the top module, I connected them all in the top module.

Receiver is normally in IDLE state. When rx_en gets logic 1, it switches to START state and the system starts to work.

In START state, it waits for half of the baud period when it detects the zero (start) bit. I measured half of the baud period with 8x baud frequency 4 times. After waiting half of the baud period, it switches to DATA state.

In the DATA state, it samples from the d_in input every 8x frequency. When it receives 8 times (when the baud frequency is ready), it compares the number of ones with the number of zeros. Whichever one is more, it outputs it. When all values in the input are read (I checked with bit_in), it switches to STOP state.

In STOP state, it waits for the baud period. Then it gives the stop bit (logic 1) and returns to IDLE state.

SIMULATION WAVEFORM

Because d_out has a value of X when the script starts running. It gives X value to the bits that are not filled until it is filled with the first 8 bits. Then it recovers. The top photo shows that it gives the result of the first data correctly. The value we give is the test_data value, we give this value in series with d_in and see it at the output in parallel with d_out.

I gave 4 different values in the top photo. The code correctly extracted them all in parallel.

FINITE STATE MACHINE

SIMULATION SOURCE

module uart_rx_tb;

parameter freq = 115200; // Baud rate

reg clk = 0;

reg rst = 0;

reg d_in = 1; // IDLE STATE

reg rx_en = 0;

wire [7:0] d_out;

wire done;

reg [7:0] test_data;

reg [7:0] expected_data;

integer file, i;

// Clock Ã¼retimi (100 MHz clock, 10 ns period)

always #5 clk = ~clk;

// Test edilen UART RX modÃ¼lÃ¼

uart_rx #(.freq(freq)) uut (

.clk(clk),

.rst(rst),

.d_in(d_in),

.rx_en(rx_en),

.d_out(d_out),

.done(done)

);

// UART veri gÃ¶nderim simÃ¼lasyonu

task send_uart_byte(input [7:0] para_in);

integer i;

begin

// Start bit (0)

d_in = 0;

rx_en = 1;

#(8640);

// Data bitleri (LSB'den MSB'ye)

for (i = 0; i < 8; i = i + 1) begin

d_in = para_in[i];

#(8640);

end

// Stop bit (1)

d_in = 1;

#(8640);

end

endtask

// Test senaryosu

initial begin

file = $fopen("stimulus2.txt", "r");

if (file == 0) begin

$display("Error: Cannot open stimulus2.txt");

$finish;

end

$display("Starting UART RX testbench...");

rst = 1;

#10000;

rst = 0;

rx_en = 0;

#45;

i = 0;

while (!$feof(file)) begin

$fscanf(file, "%b\n", test_data);

expected_data = test_data;

rx_en = 1;  // RX etkinleştir

send_uart_byte(test_data);

#100;

rx_en = 0;  // Yeni veri için devreyi sıfırla

#60;

if (d_out == expected_data) begin

$display("Test %0d Passed! Sent: %b, Received: %b", i + 1, expected_data, d_out);

end else begin

$display("Test %0d Failed! Sent: %b, Received: %b", i + 1, expected_data, d_out);

end

i = i + 1; // Sayaç doğru artırılıyor

end

$display("Test completed!");

$fclose(file);

$finish;

end

endmodule

I created a task to test the receiver in the simulation code. I tested it by taking 4 data from the file I created with the Phyton code. I compared the result I obtained with the result that should be and printed it to the console.

TOP MODULE

module uart_top #(parameter [16:0] freq = 115200) (

input clk,

input rst,

input [7:0] data_in_tx,

input tx_en,

input rx_en,

output [7:0] data_out_rx,

output tx_done,

output rx_done,

output tx_start,

output rx_start,

output tx_busy,

output rx_busy,

output tx_out

);

wire tx_baud_en,tx_count_baud_ready;

wire rx_baud_en,rx_count_baud_ready,rx_count_8x_ready;

// RX için baud gen modülü

baud_gen #(.freq(freq)) rx_baud_gen (

.clk(clk),

.rst(rst),

.baud_en(rx_baud_en),

.count_8x_ready(rx_count_8x_ready),

.count_baud_ready(rx_count_baud_ready)

);

// TX için baud gen modülü

baud_gen #(.freq(freq)) tx_baud_gen (

.clk(clk),

.rst(rst),

.baud_en(tx_baud_en),

.count_8x_ready(),

.count_baud_ready(tx_count_baud_ready)

);

// UART alıcı modülü (RX)

uart_rx #(.freq(freq)) rx_inst (

.clk(clk),

.rst(rst),

.d_in(tx_out),

.rx_en(rx_en),

.baud_en(rx_baud_en),

.count_8x_ready(rx_count_8x_ready),

.start(rx_start),

.busy(rx_busy),

.count_baud_ready(rx_count_baud_ready),

.d_out(data_out_rx),

.done(rx_done)

);

// UART verici modülü (TX)

uart_tx #(.freq(freq)) tx_inst (

.clk(clk),

.rst(rst),

.d_in(data_in_tx),

.tx_en(tx_en),

.count_baud_ready(tx_count_baud_ready),

.start(tx_start),

.busy(tx_busy),

.baud_en(tx_baud_en),

.seri_out(tx_out),   // Seri çıkış

.done(tx_done)       // Verici işlem tamamlandı sinyali

);

endmodule

DESIGN SOURCE

I instantiate a transmitter, a receiver and two baud_gen. A baud_gen for receiver other for transmitter. I made all connections in top module. In the top module, I receive 8-bit parallel data and give it to the input of the transmitter. I give the output of the transmitter to the input of the receiver. I give the output of the receiver to the output of the top module.

SIMULATION SOURCE

I give the file I created in Phyton to the transmitter input. Finally, I compare the data_out I get from the receiver with the data from the file.

module uart_top_tb();

reg clk = 1'b0;

reg rst = 1'b0;

reg [7:0] data_in_tx;

reg tx_en = 0;

reg rx_en = 0;

wire [7:0] data_out_rx;

wire tx_done;

wire rx_done;

wire tx_busy;

wire rx_busy;

wire tx_start;

wire rx_start;

wire tx_out;

integer  i;

// UART top modül örneği

uart_top uut (

.clk(clk),

.rst(rst),

.data_in_tx(data_in_tx),

.tx_en(tx_en),

.rx_en(rx_en),

.data_out_rx(data_out_rx),

.tx_done(tx_done),

.rx_done(rx_done),

.tx_busy(tx_busy),

.rx_busy(rx_busy),

.tx_start(tx_start),

.rx_start(rx_start),

.tx_out(tx_out)

);

integer file;

reg [7:0] stimulus_data[0:3];

// Saat sinyali üretimi

always #5 clk = ~clk;  // 10ns periyotlu (100 MHz) saat

initial begin

file = $fopen("stimulus2.txt", "r");

if (file == 0) begin

$display("Error: Cannot open stimulus2.txt");

$finish;

end

for (i = 0; i < 4; i = i + 1) begin

$fscanf(file, "%b\n", stimulus_data[i]);

end

$fclose(file);

rst = 1;

#20000;

rst = 0;

tx_en = 0;

rx_en = 0;

for (i = 0; i < 4; i = i + 1) begin

data_in_tx = stimulus_data[i];

tx_en = 1;

#10;

tx_en = 0;

rx_en = 1;

wait(tx_done);

#1000;

if (data_out_rx == stimulus_data[i]) begin

$display("Data received correctly: %h", data_out_rx);

end else begin

$display("Error: Received data %h, expected %h", data_out_rx, stimulus_data[i]);

end

#20000;

end

$finish;  // Test tamamlandı

end

endmodule

SIMULATION WAVEFORM

I receive the data from the file with data_in_tx and receive it from the transmitter. I give tx_out to the output in serial. At the same time, I give the tx_out output of the transmitter to the input of the receiver. Finally, the data coming in with data_in_tx comes out of the receiver correctly as data_out_tx.

BLOCK DIAGRAM

UTILIZATION REPORTS

Our WHS value is 5.839 ns.  Therefore, 10 – 5.839 = 4.161 ns. Maximum clk frequency is

1/ 4.161 ns = 240 MHz

POST – TIMING IMPLEMENTATION SIMULATION

Top Module Simulation

Baud Generator Simulation

Receiver Simulation

Transmitter Simulation

REFERENCES

[1]  Peňa, E., & Legaspi, M. G. (n.d.). UART: A Hardware Communication Protocol Understanding Universal Asynchronous Receiver/Transmitter. Analog Devices. https://www.analog.com/en/resources/analog-dialogue/articles/uart-a-hardware-                                   communication-protocol.html

[2]  Kelvin. (2022, December 15). UART Communication Protocol and how it works - latest open tech from SEEED. Latest Open Tech From Seeed. https://www.seeedstudio.com/blog/2022/09/08/uart-communication-protocol-and-how-it-works/

[3]  01signal: Vivado: Finding the “maximal frequency” after synthesis. (n.d.). https://www.01signal.com/vendor-specific/xilinx/vivado-minimal-period/

[4]  [Mehmet Burak Aykenar]. (2021, January 11). VHDL ile FPGA PROGRAMLAMA - Ders18: UART Receiver Tasarımı ve Simülasyonu - Sıfırdan Kod Yazma [Video]. Youtube. https://www.youtube.com/watch?v=d7XPkZDWWQY&t=1052s

[5] [[Mehmet Burak Aykenar]. (2021, January 8). VHDL ile FPGA PROGRAMLAMA - Ders16: VHDL UART Transmitter Sıfırdan Kod Yazma [Video]. YouTube. https://www.youtube.com/watch?v=5MzjKMZfLTk&t=1904s

[6]  [Muhammed Kocaoğlu]. (2023, April 2). SystemVerilog - UART Transmitter [Video]. Youtube. https://www.youtube.com/watch?v=bh_RxlzEnvU&t=757s

[7]  [Muhammed Kocaoğlu]. (2023, April 14). SystemVerilog - UART Receiver [Video]. Youtube. https://www.youtube.com/watch?v=va-q8QKqeMg&t=596s